# Maintenance Report S02

Document ID: MR-321 | Title: Maintenance Report S02 | Factory: FACTORY-DEMO-01 | Station: S02 | Classification: CONFIDENTIAL | Revision: 1.1 | Effective Date: 2026-01-17 | Owner: Demo Operations Engineering

# Work Order

MT-S02-20260117 | Technician: Demo Technician A. Novak | Downtime: 42 minutes

# Trigger and initial diagnosis

Repeated POSITION-ENC-02 warnings on P4801, P4802, P4805 and P4811. Encoder feedback was unstable during homing.

# Findings

Connector seating was acceptable; encoder signal drift exceeded the demo maintenance tolerance.

# Corrective work

Encoder replacement, controlled homing, three dry cycles, and S04 inspection verification completed.

# Measurements

Before: reference deviation 0.84 mm. After: reference deviation 0.06 mm.

# Return to service

S02 returned to service after dry-cycle and inspection verification. Final status: CLOSED.

# Sign-off

Demo Technician A. Novak | Demo Operations Engineering | Revision 1.1